Simple stresses and strains

Definition:

Rigid body: a rigid body is one which is not deformed under the action of various forces.

Plastic body: a plastic  body is one which do not show any tendency to recover their original form after deforming forces removed.

Mechanical properties

- Elasticity : It is the property by virtue of which a material deformed under the load is enabled to return to its original dimension when load is removed .

Example:steel,aluminium,copper,concrete  etc.

- Plasticity : it is the ability to undergo some degree of permanent deformation without rupture or failure.

- Compressibility :  defined as the property by which a metal resists the action of a compressive force.

- Hardness: it is the ability of a material to resist indensation or suface abrasion.

- Toughness: it  is the property of material which enables it to absorb energy without fracture.

- Stiffness: stiffness is a qualitative measure of the elastic deformation produced in a material. a stiff material has a high modulus of elasticity.

- Brittleness: a material is said to be brittle when it cannot be drawn out by tension to smaller section.

- Ductility :it is the characteristics  which permit a material to be drawn out longitudinally to a reduced section.

- Malleability: property of material which permits the materials to be extended in all directions without rupture.

- Creep: it is the tendency of a solid material to slowly move or deform permanently under the influence of  stresses.

- Fatigue : if material subjected to repeated, variable loads of opposite direction,then it fails at load lesser than ultimate load . This property is called fatigue of a metal.

- Tenacity : property by  which a metal resists the action of a pulling force.

Simple stress  and strain

Stress:

The force of resistance per unit area, offered by a body against deformation is known as stress.

stress=Applied force  cross sectional area

σ=pA

where σ=stress

P=Applied load

A=Cross sectional area .

Unit:N/m2

Strain :

The ratio of change in dimension of the body or deformation to the original dimension of the body  is known as strain.

strain,ε=change in dimension of the body or deformationorginal  dimesion of the body

ε=δll

TYPES OF STRESSES :

- Normal stress

- Tensile stress

- Compressive stress

- Shear stress or Tangential stress

- Bearing stress

- Bending stress

- Twisting or torsional stress.

Types of strain

- Tensile strain

- Compressive strain

- Shear strain

- Volumetric strain

- Superficial strain

AXIAL LOAD

The force or load acting along the axis of the bar or body is known as axial load.

Normal stresses:

the internal forces and the corresponding stresses acts in a direction perpendicular to the surface is known as normal stresses  or direct stress .

normal stresses two types

- Tensile stress

- Compressive stress

Tensile stress

It is a stress induced in a body when it is subjected to two equal and opposite pulls(tensile force) as a result of which there is tendency in increase in length.it acts normal to the area and pulls on the area

Fig shows a bar subjected to a tensile force at its end.consider section x-x as shown in fig. the portion left or right of the section is in equilibrium if resisting force p’= applied  tensile force P . this resisting force per unit cross sectional area is known as tensile stress or unit stress

Let P=Applied tensile load ,P’=Resisting force

Tensile stress=Resisting forcecross sectional area =Applied tensile load cross sectional area

σ=PA                 (∵P'=P)

Tensile strain:

The ratio of increase in length to the original length is known as tensile strain.

Consider a bar subjected to tensile load or force P

Let AB=original length of bar -l

AC= final length of bar after the application of tensile load P

∴Extension of bar=AC-AB=BC=δl

Tensile strain=Extension  in length of baroriginal length of bar

ε=δll

Compressive stress

Stress induced in a body , when subjected two equal and opposite pushes as a result of which there is tendency of decrease in length of the body.it acts normal to the area and it pushes on the area.

Compressive stress=Resisting forcecross sectional area =Applied compressive load cross sectional area

σ=PA                 (∵P'=P)

Fig shows a bar subjected to compressive  load  at its end . consider section x-x as shown in fig the portion left or right section is in equilibrium if resisting force P’= applied compressive force P. this resisting compressive force per unit cross sectional area is known as compressive stress or unit stress.

Compressive strain

The ratio of decrease in length to the original length is known as compressive strain.

Let AB=original length of bar -l

AC= final length of bar after the application of tensile load P

∴shortening  of bar=AB-AC=δl

Compressive strain=Decrease in length of bar original length of bar

Compressive strain ,             ε=δll

d=diameter of bolt

t=Thickness of plate

shear stress:

when a body subjected to two equal and opposite forces, acting parallel or tangential across the resisting sections and tends to shear off the body across the section then the force is called shear force and stress induced is called shear stress. Corresponding strain is shear strain

Shearing strain

Fig  shows a rectangular block of height h,length l and unit width is subjected to shear force P on the top and bottom faces. As the bottom face is fixed , the block deformed to the position ABC1D1 due to the applied shear force P as shown in fig

Let  δl=horizontal displacement of the upper face

h =height of block

shear strain=φ=δlh(∵φ is small ,tanφ≈φ  )

Elastic limit: is the maximum stress from which an elastic body will recover its original state after the removal of the deforming force.

Hooks law:

Hooks law states that within elastic limit stress is directly proportional to strain

stress∝strain

or stressstrain=a constant

this constant is known as coefficient of elastiacity or modulus of elasticity.

Bearing stress : It is a localized compressive stress induced at the surface of contact between two members of a machine part that are relatively at rest.

Consider two plates connected by a bolt as shown in fig .the bolt exerts  aresisting force P’=,equal and opposite to the forces P exerted by the plate on the bolt.

Bearing  stress=Resisting force exerted by the bolt on plate projection area of bolt on the plate  section=Force  exerted by the plate on the bolt projection area of bolt on the plate  section

σb=Pdt                 (∵P'=P)

Where d=diameter of bolt

t= thickness of plate

Elastic constants

Youngs modulus(E) : The ratio between tensile stress and tensile strain or compressive stress and compressive strain is called young’s modulus or modulus of elasticity. It is denoted by E

E=Tensile stress Tensile strain

or
E=Compressive stress compressive strain

E=σε

2 .Rigidity modulus(C):The ratio between shear stress and shear strain is called shear modulus

Or rigidity modulus .it is denote by C or G or N

C=Shear stress shear strain

3.Bulk modulus(K): The ratio of direct stress to the corresponding volumetric strain is known as bulk modulus . It is denoted by K

K=stress Volumetric strain

2.11 Extension or elongation and shortening of bar

When a bar subjected to tensile load ,it will elongates δl similarly if bar subjected to compressive load ,it will shortened by amount δl.

Let, P=Applied load

A= Area of cross section of bar

L=original length of bar

δl=change in length

Stress,σ=pA

Strain,ε=δll

E=linear stress linear  strain

E=σε

E=PAδll

E=PlAδl

Extension or shortening of bar 			δl=PlAE

Saint venants principle

This priciple states that except in the region of extreme ends of a bar carrying direct loading ,the stress distribution over the cross section is uniform.

Consider  a slender bar with point loads at its end as shown in fig .the normal stress distribution across section located at distance b/4  and b from one end of the bar is represented in fig.

The points very near the application of the loads experience a large stress value whereas, the points far away from it on the same section has lower stress value.

The variation of stress across the cross section is negligible when the section considered is far away, about equal to the width of the bar. from the application of point loads.

Stress strain diagram

A stress strain curve for mild steek is shown in fig.

The portion OA of the graph is a straight line , which obeys Hook’s law . i.e., stress being proportional to strain. The point A Corresponds to the limit of proportionality. At B mild steel begins to yield and corresponding stress is called yield stress.the yielding continuous upto C  and portion BC is nearly parallel to OX. From C to D the material is ductile and there is a general reduction of the area of cross section of the test specimen. At point E the stress is maximum and is known as ultimate stress.beyong E

Limit of proportionality (A): It is the limiting value of the stress up to which stress is proportional to strain.

Elastic limit: This is the limiting value of stress up to which if the material is stressed and then released (unloaded) strain disappears completely and the original length is regained.

Upper yield point(B): This stress at which the load starts reducing and extension increases . this phenomenon is called yielding of material . at this stage strain is about 0.125 and stress is about 250N/mm2.

Lower yield point (C) : At this stage the stress remains same but strain increases for some time.

Ultimate stress: This is the maximum stress the material can resist. At this stage cross sectional area at a particular section start  reducing very fast . this is called neck formulation. After this stage load resisted and hence the stress developing start reducing

Breaking point(F): The stress at which finally the specimen fails is called breaking point.

Gauge length: It is the portion of the test specimen over which extension or deformation is measured.

Stress levels

- Yield stress or Nominal stress: It is the ratio of the yield load to the original area of cross section of test specimen.

- Working stress: the maximum load to which a member likely to be subjected during its working period is called the working load. And the maximum allowable stress in the material is called working stress or permissible stress.

- Ultimate stress: the maximum load to which a bar is subjected to in a test divided by its original cross sectional area gives a nominal stress which is known as ultimate stress or maximum stress.

- Breaking stress: the stress corresponding to fracture load is called breaking stress.

Stress strain relation for other materials including nonferrous metals.

Stress strain curves for steek having carbon content varying from 0.12% to 1% . from these it is concluded that with high carbon content it is similar to brittle materials even though the ultimate stress is much higher.

Stress – strain diagram for common materials such as  high carbon steel, nickel chrome steel, mild steel, wrought iron, cast iron, copper and cast aluminum . from these it is noticed that for steel and wrought iron proportionality exists almost until yielding takes place.

For copper, cast iron, aluminum and high alloy ,no clearly defined limit of proportionality ,elastic limit or yielding point. Cast iron behave like a brittle material which fails whithout any visible elongation or reduction area.

Fig shows stress strain diagram for aluminum alloy. It exhibits considerable ductility though it does not have clearly definable yield point.

Fig shows stress strain diagram for hard and soft rubber. The curve for hard rubber is linear upto very large strains .soft rubber continues to stretch enormously without fracture.

Stress strain relation for brittle material(cast iron)

It is seen from the diagram that the deviation from hooks law begins very early and fracture occurs suddenly with very small deformation and without necking . the stress strain diagram for brittle material does not exhibit the yield point .

Examples :stone ,glass,concrete

Percentage elongation and percentage reducation in area .

Percentage elongation is the percentage increase in length of the gauge length

∴Percentage elongation =Lf-LoLo

Where Lo=Original gauge length

Lf=Final gauge length at rupture.

Percentage reduction in area is defined as the ratio of maximum change in cross -sectional area to the original cross sectional area, exposed in percentage .

∴Percentage elongation =Ao-AfAo

Where Ao=Original cross sectional area

Af=cross sectional area in waist at fracture .

- Materials with more than 15% elongation are usually considered as ductile

- Materials with less than 5%elongation are considered as brittle

- Many ductile materials have the same yield point in tension and compression

- Most brittle materials have a higher value  for ultimate strength in compression than that for in tension

- For ductile material percentage reduction in area is 50 to 70.

FACTOR OF SAFETY

The ratio of the ultimate str

ess and working stress for a material is called factor of safety.

Factor of safety =Ultimate stressWorking stress

Factor of safety depends on many considerations

- The nature of loading

- The homogeneity of the materials used

- The accuracy with which stresses in members and external forces can be evaluated

- The degree of safety required

- The degree of economy desired

Load factor:

Load factor =Load at failureWorking load

Linear strain and lateral strain

A rectangular bar under longitudinal tension extend along its length but at the same time there will be a shortening of its breadth as well as thickness.

The longitudinal or linear strain defined as deformations of the body per unit length in the direction of the applied load.

The lateral strain defined as deformations of the body per unit breadth or thickness in the perpendicular direction of the applied load.

longitudinal strain=δll

lateral  strain=δbb or δtt

where δb & δt are the change in breadth and change in thickness respectively.

b and t are original breadth and thickness respectively.

POISSON’S RATIO

The ratio between the lateral strain and the longitudinal strain is called Poisson’s ratio and is denoted by 1m or μ. It lies between 0.33 and 0.25

lm=Lateral strain  Longitudinal strain

VOLUMETRIC STRAIN :

volumetric strain ,εv=Change in volume  original volume=δVV

εv=ε(1-2m)

RELATION BETWEEN ELASTIC CONSTANTS

- RELATION BETWEEN MODULUS OF ELASTICITY AND BULK MODULUS

Let ,E=modulus of elasticity

K= bulk modulus

1m=Poisson's ratio

Now E=3K(1-2m)

Or K=mE3(m-2)

- RELATION BETWEEN MODULUS OF ELASTICITY AND SHEAR MODULUS

Let ,E=Modulus of elasticity

c= Rigidity modulus

1m=Poisson's ratio

Now E=2C(1+1m)

Or K=mE2(m+1)

- REALTION BETWEEN MODULUS OF ELASTICITY ,MODULUS OF RIGIDITY AND BULK MODULUS

Let ,E=modulus of elasticity

K= bulk modulus

1m=Poisson's ratio

c= Rigidity modulus

K=9GKG+3K

ELONGATION OF BARS OF VARYING CROSS SECTIONS

Consider a bar of varying three sections of length l1,l2 and l3 having respective areas of cross sections A1,A2  &A3  subjected to an axial pull P .let δl1, δl2 and  δl3 be the changes in length of the respective three sections of the bar then we have .

Elongation , δl=PlAE

∴δl1=Pl1A1E

δl2=Pl2A2E

δl3=Pl3A3E

NOW ELONGATION OF BAR

∆l=δl1+δl2+δl3

∆l=Pl1A1E+Pl2A2E+Pl3A3E

∆l=PEl1A1+l2A2+l3A3

APPLICATION OF PRINCIPLE OF SUPERPOSITION TO DETERMINING ELONGATION

While number of loads acting on a body, the resulting strain, according to the principle of superposition, will be algebraic sum of strains caused by individual loads .

While using this principle for elastic body subjected to a number of direct forces,tensile  or compressive at different sections along the length of the body . the total deformation will be equal to the algrebraic sum of deformation of the individual section .

∆l=1AEP1l1+P2l2+P3l3

+sign for tensile load

-sign for compressive load .

1.An alloy Bar 1m long and 20mmx10mm in cross section is subjected to a compressive load of 20kN. If the modulus of elasticity for the alloy is 100Gpa, find the decrease in length of the bar

Given : 	length of  bar ,l=1m=1000mm

Compressive load ,P=20 kN=20×103 kN

Modulus of elasticity ,E=100Gpa=100×109N/mm2=100×103N/mm2

A=20mm×10mm=200mm2

Solution:

Decrease in length of bar ,    δl=PLAE=20×103×1000200×100×103=1mm

- a straight bar of 50mm length has its cross sectional area of 500mm2 .Find the magnitude of the compressive load under which it will decrease its length by 0.2mm. Take E  for the bar material as 200Gpa.

Given:   l=500mm

A=500mm2

δl=0.2mm

E=200Gpa=200109×N/m2=200×103N/mm2

Solution:

Strain,	ε=δll=0.2500=4×10-4

Young’s modulus , E=σε

i.e,			200×103=σ4×10-4

σ=80N/mm2

σ=PA

80=P500

P=80×500=40000N

Compressive load,       P=40 kN

- A short column has an internal diameter of 200mm. what should be the minimum external diameter so that it may carry a load of 1600 kN. With a factor of safety 5?ultimate stress of steel is 472 N/mm2

Factor of safety=ultimate stressworking stress

1.75=250working stress

working stress=2501.75

Working stress=142.86N/mm2

.. .	σ=142.86N/mm2

Stress, σ=PA

142.86=1600×103A

A=1600×103142.86

A=16949.15mm2

A=π(do2-d12)4

16949.6=π(do2-2002)4

16949.16=0.786(do2-2002)

or	0.786do2-2002=16949.6

do2-2002=16949.60.786

do=16949.60.786+2002

do=248.12mm

- A rectangular bar 1000mmx50mmx40mm long is subjected to an axial pull of 250 kN. If poisson’s ratio is 0.3 and E=2x105N/mm2. Find the change in length ,width, thickness and volume.

Given :b=50mm, t=40mm,l=1000mm,P=250 kN=250000N,

1/m=0.30,E=2×105N/mm2.

Area =A=b×t=50×40=2000mm2.

σ=PA=2500002000=125N/mm2

E=σε

2×105=125ε

ε=1252×105=6.25×10-4

ε=δll

Elongation , δl=ε×l=6.25×10-4×1000=0.6255incresing

1/m=εlε

εl=1m×ε=0.3×6.25×10-4=1.875×10-4

Change in breadth

εl=δbb

δb=bεl=1.875×10-4×50=9.375×10-3mm

Change in thickness

εl=δtt

δb=tεl=1.875×10-4×40=7.5×10-3mm

Change in volume

Volumetric strain ,

εv=ε1-2m=6.25×10-41-2×0.3=2.5×10-4

Volume,v=l×b×t=1000×50×40=2×106mm3

∴change in volume,δv=εv×v=2.5×10-4×2×106=500mm3(increase)

- In an experiment a bar of 30m diameter is subjected to a pull of 60kN . The measured extension on gauge length of 200mm is 0.09 mmand change in diameter is 0.039 mm .calculate poisson’s ratio and the values of the three moduli.

Given d=30mm, P=60 kN=60000N, l=200mm, δl=0.09mm, δd=0.0039mm

Area,

A=πd24=π×3024=706.85mm2

∴σ=PA=60000706.85=84.87N/mm2

ε=δll=0.09200=4.5×10-4

E=σε=84.874.5×10-4

εl=δdd=0.003930=1.3×10-4

1m=εlε=1.3×10-44.5×10-4=0.28

E=2C1+1m

188600=2×C1+0.28

2×C1+0.28=188600

C=1886002(1+0.28)

Rigidity modulus , C=73672N/mm2

E=3K1-2m

188600=3×K1-2×0.28

3×K1-0.28=188600

K=1886003(1-0.28)

Bulk modulus,  K=142878N/mm2

- A steel rod subjected to force shown in fig below fing the elongation of the rod

Solution: given , l1=400mm, l2=600mm, A2=20x20=400mm2, P=40 kN=40×103N

E=200Gpa=200 ×103N/mm2

A1=πd124=π×4024=1256.63mm2

∆l=PEl1A1+l2A2

∆l=40×103200×1034001256.63+600400

∆l=0.364

- A brass bar having cross sectional area of 1000sq.mm is subjected to axial forces as shown in fig.

Find the total elongation of the bar take E=80 kN/mm2

Solution: