TREATMENT OF WATER

Water available in various sources contains various types of impurities and cannot be directly used by the public for various purposes, before removing the impurities for portability water should be free from unpleasant tastes, odours and must have sparkling appearance. The water must be free from disease-spreading germs. The amount and type of treatment process will depend on the quality of raw water and the standards of quality of raw water.

The treatment of water includes the following purposes,

- To remove larger, heavy and floating objects for easy flow of water.

- To remove the elements responsible for turbidity, colour, odour and make water aesthetically attractive,

- To remove unpleasant tastes for making water palatable.

- To remove pathogenic bacteria and other disease causing germs.

- To control concentration of minerals such as calcium, fluoride and iodine.

- To prevent corrosion, tuberculation and incrustation in pipes.

Flow diagram of different units of treatment:

1) SCREENING

Screens are mechanical devices, which are provided before the intake works so as to prevent the entry of floating matters of comparatively large size. If such materials are not removed, they will choke up the small pipes or affect seriously on working of other water treatment units.

Depending on the quantity of water from a source, any of the following three types of screens may be adopted.

1) Fine screens

2) Medium screens

3) Coarse screens.

1) Fine screens: Fine screens permit particles of size less than 6mm to flow through them. These screens have perforations of size about 1.5 mm to 6mm and they considerably reduce the load on subsequent treatment units. The fine screens are generally made of metal plates and wire mesh. The metal should be resistant to rust and corrosion.

2) Medium screens:  Medium screens have a size of opening about 25mm to 75 mm. these screens are most commonly are most commonly used in water treatment. 6mm thick and 25 mm wide rectangular bars, inclined at 30 to 60 to the horizontal, the inclination facilitates easy cleaning on the upstream side of the screen.

3) Coarse screens:  Coarse screens have the spacing between the bars of about 75 mm to 100 mm. these screens are commonly adopted for the treatment of waste water and rarely for water treatment. The solids retained by the screens are predominantly organic, inert and harmless, which can be easily disposed off.

2) PRE-SEDIMENTATION

When water is highly turbid, further treatment as aeration becomes difficult. Hence, to reduce this suspended load, sedimentation precedes aeration and this process is called pre-sedimentation.

Pre-sedimentation tanks are similar to regular sedimentation tanks but are of small in size.

3) AERATION

This is the process of bringing water into intimate contact with air. Aeration is done to remove taste and odour and aeration is necessary to promote the exchange of gases between the water and the atmosphere.

Water aeration is the process of increasing or maintaining the oxygen saturation of water in both natural and artificial environments.

Objects of aeration:

- To add oxygen to water for imparting freshness, for example water from underground sources may have lesser oxygen.

- Drive out objectionable dissolved gases like carbon dioxide, hydrogen sulphate and substances causing taste and odour.

- To precipitate impurities like iron and manganese especially from underground water.

- Due to agitation of water in the process of aeration, the bacteria may be killed to some extent.

TYPES

Following are the five methods of aeration which are generally adopted in the treatment of water:

1) Air diffusion

2) Cascades

3) Spray nozzles

4) Tray tower

5) Trickling beds.

1) Air diffusion:

In this method, the perforated pipes are installed at the bottom of tanks. The compressed air is blown through these pipes. The air bubbles while coming up from the bottom of tank come into close contact of water contained in the tank and aeration of water is achieved. The depth of aeration tank is generally kept as about 2.5 m to 3 m and they work on the principle of continuous flow with minimum detention period of 15 minutes.

2) Cascades or tray tower:

In cascade aerators water is allowed to flow downwards after spreading over inclined surface in thin sheets and the turbulence is secured by allowing the water to pass through a series of steps or baffles. The number of steps is usually 4 to 6. Exposure time can be increased by increasing the number of steps. The steps are spaced at a distance of about 250 mm to 750 mm. this method of aeration is much more effective and remove CO2 to the extent of about 90 to 95%.

3) Spray nozzles:

In this method, the water is sprinkled in fine jets through nozzles to a height of about 2 m to 2.5 m, having diameter of 20 mm to 40 mm and spacing of about 0.5 m to 3.5 m. discharge of water through a nozzle is at a rate of about 5 to 10 l/sec. this method of aeration remove carbon dioxide to the extent of about 90%. But it requires considerable head of water for its working.

4) SEDIMENTATION

The particles which are heavier than the water are naturally likely to settle down due to the force of gravity.

In water, there are mainly two types of impurities, inorganic suspended solids having specific gravity about 2.65 and organic suspended solids having specific gravity of about 1.04. The particles having specific gravity of about 1.20 or so easily settle down at bottom of tank. But it is difficult to cause the settlement of lighter particles. This phenomenon of settling down of particles at the bottom of sedimentation tank is known as hydraulic subsidence and every particle has its own hydraulic settling value which will cause hydraulic subsidence. But the lighter particles cannot settle down due to force of gravity. Such particles are converted to settleable size by application of some coagulant in water. The velocity of flow can be decreased by increasing length of travel and thus a particle is allowed to stay for longer period in sedimentation tank.

SEDIMENTATION TANKS

1) Plain sedimentation

a)  Fill and draw type sedimentation tanks

b) Continuous flow type sedimentation tanks.

c) Circular tanks with radial or spiral flow.

2) Sedimentation aided with coagulation.

1) Plain sedimentation

When the impurities are separated from the suspending fluid by gravitation and natural aggregation, the operation is called plain sedimentation.

Advantages

1) It is a preliminary process which lightens the load on subsequent process.

2) It gives less variable quality of water and so the operation of subsequent purification process is done

in a better way.

3) No chemical is lost with the sludge discharged from the plain settling tanks

4) Cleaning cost of chemical coagulating basin is reduced.

5) Less quantity of chemicals are needed in the subsequent treatment processes.

Commonly used sedimentation tank is a rectangular type, but circular type of tank is also used. Sometimes the spiral type of tank may use for sedimentation.

Detention period The theoretical time taken by a particle of water to pass between entry and exit of a settling tank is known as the detention period.

a) Fill and draw type or intermittent sedimentation tanks:

- The working of this tank is simple. The water is filled in the tank and it is allowed to rest for 24 hrs.

- During the period of rest, the particles in suspension will settle down at the bottom of tank.

- The clear water is then drawn off and the tank is cleaned of silt and filled again.

- If time required for inlet, outlet, emptying and cleaning operation is of about 30 to 36 hrs.

- This means that, at least two tanks will be required and if an additional unit is to be provided as stand-by.

- Minimum three numbers of tanks is required for this type of working.

b) Continuous flow type sedimentation tanks

- The working of the tank is very simple. The water enters the tank from one end and it travels towards the outlet at the other end.

- Its velocity is broken or reduced by means of baffle walls.

- The walls contain openings at different levels.

- The velocity of flow is so adjusted that time taken by a particles of water to move from one end to the other is slightly more than that required for the settlement of suspended impurities in water.

- The entry of impure water from one end and the exit of clear water from the other end are continuous.

- The velocity of flow is such that suspended impurities of required size settle down at the bottom of tank.

- The total amount of flow from the tank within 24 hrs is equal to the daily demand of water.

- The silt is deposited at the bottom of tank and when it is accumulated in sufficient quantity, the flush valve is opened and the tank is cleaned.

c) Circular tanks with radial or spiral flow

- The circular settling tanks are costly to install, but they are convenient when mechanical sludge scrapers are used to remove sludge from the bottom of settling tank.

- It may have radial or spiral flow, Dorr clarifier which is a circular sedimentation tank with radial flow.

- In this type of tank, the water is admitted from the centre of tank and is taken out at the circumference through drawoff channel.

- The impurities settle at the bottom of tank. The raking arms are slowly rotated and blades of raking arms lead the mass of impurities to the sludge discharge pipe.

- The water is allowed to move spirally in the tank and then it is taken out through a submerged outlet.

2) Sedimentation aided with coagulation

When chemical or other substances are added to increase aggregation and settling of finely divided suspended matter and colloidal substances, the operation is called sedimentation with coagulation.

OR

The fine suspended particles like mud particles and the colloidal matter present in water cannot settle down by plain sedimentation with ordinary detention periods. Some of the colloidal impurities will not settle even if the water is detained for long periods in the sedimentation.

- Certain chemicals are added to the water so as to remove such impurities which are not removed by plain sedimentation. These chemical compounds are called coagulants.

- The process of adding certain chemicals to water in order to form floc (insoluble gelatinous substance) for quick sedimentation and rapid removal of fine particles is called sedimentation with coagulation.

- When coagulants are mixed with water thoroughly, a thick gelatinous precipitate is formed which is known as floc. This floc has got the property of attracting the suspended impurities in water and settle down towards the bottom of tank.

- The phenomenon of formation of floc is termed as flocculation. The efficiency of flocculation depends on dosage of coagulant, type of mixing and pH value of water.

- Common coagulants are aluminium sulphate, ferrous sulphate and Lime, Chlorinated coppers, Sodium aluminate.

- Coagulants are chosen depending upon the pH of water. Alum or Aluminium sulphate is normally used in all treatment plants because of the low cost and ease of storage over a long periods.

Common coagulants

The common chemicals generally used for coagulation are

1) Aluminium sulphate

2) Ferrous sulphate and lime

3) Chlorinated copperas

4) Sodium aluminate

Coagulants are chosen depending upon the PH of water. Alum or Aluminium sulphate is normally used in all treatment plants because of the low cost and ease of storage as solid crystals over long periods.

1) Aluminium sulphate

It is commonly known as alum. The chemical composition of aluminium sulphate is Al2(SO4)3, 18H2O.

When alum is added to raw water, it reacts with the bicarbonate alkalinities, which are generally present in raw water and forms a gelatinous precipitate (floc) of aluminium hydroxide. This floc attracts other fine particles and suspended matter and thus grow in size and settle down finally to the bottom of the tank.

Advantages of using alum as a coagulants are as follows

- It produces crystal clear water.

- It gives better floc formation.

- The floc formed is quite tough and it is not broken easily.

- Promotes better settling and longer filter runs.

- Aids in reduction of tastes and odours.

- It is cheap.

- It is superior in tests against other coagulants.

- It is high in quality.

- It is simple in working and does not require skilled supervision.

MIXING DEVICES

After adding coagulants in water, the next step is to mix the coagulants thoroughly in water so that it gets dispersed in the whole water.

This mixing can be done by the following devices

1) Mixing basins with baffle walls

2) Mixing basins with mechanical means

a) Flash mixer

b) Flocculator

1) Mixing basins with baffle walls

In this type, the mixing tanks with baffle walls are provided to mix raw water with coagulant. These may be horizontal or vertical type. In case of horizontal type, a rectangular tank is divided into continuous series of channels by means of baffle walls which cause the water to which the coagulant have been added to flow round the ends of the baffle walls through number of channels. In case of vertical type, water flows up and down by the action of vertically hanging baffle walls.

2) Mixing basins with mechanical means

Mechanically Agitated Mixing Basins are those in which water is subjected to a violent agitation when the chemical is first added to water and then to a gentler action when the floc is forming. The first of these operations is usually obtained in Flash Mixers and the second in a Flocculator.

Flash Mixer:

In a Flash Mixer the rapid mixing is brought about in a rectangular tank, by number of revolutions of a propeller fixed to a propeller shaft and driven by an electric motor. The coagulant brought by coagulant feeding pipe and is discharge under the impeller. The raw water is brought from the inlet pipe and is deflected towards the impeller by the deflecting wall. The thoroughly mixed water finally goes out from outlet.

FLOCCULATION:

The floc formation by the action of coagulants with water is heavy and hence, it starts to settle down at the bottom of tank, As it descending, it absorbs and catches more and more suspended impurities present in water. It thus goes slowly on increasing in size. During this process, some amount of bacteria removal also takes place.

Flocculator

When the thorough mixing of coagulants in the water is over, the next operation is flocculation. The device by means of which floc is formed is called flocculator. And the phenomenon is termed as flocculation.

Flocculator essentially consists of a circular tank equipped with paddles revolving on a vertical shaft. The paddles operate at a slow speed of 2 to 3 r.p.m. the detention period for floc chamber is about 15 to 40 minutes.

Dosing

The process of adding of the coagulants in correct amount is known as dosing. This depends upon the turbidity of water, its colour, pH value, temperature of the water and time of settlement.

Calculation of optimum coagulant dose

The jar test is the laboratory method to determine the optimum dosage of a particular coagulant which is required to be added to the raw water for coagulants and subsequent sedimentation in a treatment plant.

- The jar test apparatus consists of a rotary device having six numbers vertical stirring rods provided with pedals at lower ends and called multiple stirrers.

- The water and alum content placed in six numbers of jars or beakers of 1 liter capacity.

- A horizontal shaft which is geared directly to an electric motor.

- The motor is provided with speed-reducing gears which enable the pedals to revolve at different speeds as required in the first and second stages of the coagulation process.

- The different amount of alum solution are added to each jar containing the same amount of water sample and the contents stirred rapidly at first for 2-3 minutes and slowly later for about 30 minutes.

- The liquid is finally allowed to stand so that floc may form.

- The smallest dose of alum that produces a good floc is taken as the optimum dosage for the particular water.

FILTRATION:

The process of passing the water through a thick layer of sand and gravel which acts as a strainer is called filtration. The process of filtration forms the most important stage in the purification of water. During the process of filtration, the following effects occur on water

- The suspended and colloidal impurities which are present in water in a finely divided state are removed to a great extent.

- The chemical characteristics of water are altered.

- The number of bacteria present in water is also considerably reduced.

Classification of filters

Filtering may be divided into two main classes

1) Gravity filters

a) Slow sand filters

b) Rapid sand filters

2) Pressure filters

a) Slow sand filters:

- In case of slow sand filtration, the water is allowed to pass slowly through a layer of sand placed above the base material.

- And the purification process improving the biological, chemical and physical characteristics of water.

- Slow sand filters do not have any enclosure tank. They are water tight rectangular beds of 1000 to 5000 m2 in area with a depth of about 3 to 3.5 m.

- The slow sand filtration is very well suited for rural areas because of its simple operation and maintenance procedures, and It provides safe drinking water at low cost.

A slow sand filtration consists of the following parts:

1) Under drainage system

2) Base material

3) Filter media of sand

4) Appurtenances.

1) Under drainage system

- The UDS consists of a central drain and lateral drains.

- The lateral drains are placed at a distance of about 2.50 to 3 m, and they are stopped at a distance of about 500mm to 800 mm from walls of the tank.

- The drains may be of pipes which are laid with open joints.

- The lateral drains are perpendicular to the central drain.

2) Base material

- The base material is gravel and it is placed on the top of under drainage system.

- Its depth varies from 300 to 750 mm.

- It is usually graded and laid in layers of 150 mm, the top most layer should be of small size gravel and the lowest layer should be of bigger size gravel.

- Topmost layer               150 mm depth        size 3 mm to 6 mm

Intermediate layer       150 mm depth         size 6 mm to 20 mm

150 mm depth         size 20 mm to 40 mm

Lowest layer                  150 mm depth         size 40 mm to 65 mm

3) Filter media of sand

- A layer of sand is placed above gravel. The depth of sand layer varies from 600 mm to 1000 mm.

- The effective size of sand varies from 0.20 mm to 0.40 mm.

- The depth of a good layer is 600 mm due to successive scraping of surface as a process of cleaning.

- The finer the sand, the better will be the efficiency of filter regarding removal of bacteria. But in that case, the output from filter is lowered.

4) Appurtenances

- The vertical air pipe passing through layer of sand, such pipes help in proper functioning of filtering layers.

- The device for measuring loss of head, and controlling depth of water above sand layer are to be suitably installed.

Advantages

1. Simple to construct and supervise.

2. Suitable where sand is readily available.

3. Effective in bacterial removal.

4. Preferable for uniform quality of treated water.

Disadvantages

1. The slow sand filters require large area for their construction and high initial cost for establishment.

2. The rate of filtration is also very slow.

3. Less flexibility in operation due to seasonal variations in raw water quality.

b) Rapid sand filters

- It is observed that the rate of filtration is more in coarse sand than that in fine sand.

- The theory of rapid sand filtration is based on principle of increasing the rate of filtration by providing coarse sand as filter media.

- Rapid sand filter are replacing the slow sand filters because of high rate of filtration with small area of filter.

- Water passes downwards through the media and the filters are cleaned by backwashing.

A slow sand filtration consists of the following parts:

1) Enclosure tank

2) Under drainage system

3) Base material

4) Filter media of sand

5) Appurtenances.

1) Enclosure tank

- A watertight tank is constructed either of masonry or concrete.

- The sides and floor of tank are coated with waterproof material.

- The depth of tank is about 2.50 to 3.50 m and the surface area of a filter media varies from 10 to 50 m2.

2) Under drainage system

Following are two common types of under drainage system

1) Perforated pipe system

- The UDS consists of a central drain and lateral drains.

- The lateral drains are placed at a distance of about 150 to 300 mm.

- The lateral drains are provided with holes at the bottom side of 10 mm dia with 300 angles.

- The holes are drilled at a centre to centre distance of about 75 to 200 mm.

2) Pipe and strainer system.

- In this system, there is a central drain is attached with lateral drains on either side.

- The strainers are placed on lateral drains instead of drilling holes.

- A strainer is a small pipe of brass, it is closed at top and contains holes on its surface.

- The strainers are either screwed or fixed on the top of lateral drains.

3) Base material

- The base material is gravel and it is placed on the top of under drainage system.

- Its depth varies from 450 to 600 mm.

- It is usually graded and laid in layers of 150 mm, the topmost layer should be of small size gravel and the lowest layer should be of bigger size gravel.

- Topmost layer               150 mm depth        size 3 mm to 6 mm

Intermediate layer       150 mm depth         size 6 mm to 12 mm

150 mm depth         size 12 mm to 20 mm

Lowest layer                  150 mm depth         size 20 mm to 40 mm

4) Filter media of sand

A layer of sand is placed above gravel. The depth of sand layer varies from 600 mm to 750 mm.

- The coarse sand is used as filter media.

- The effective size of sand varies from 0.5 mm to 1 mm.

- The space of voids between sand particles is increased and it results in the increase rate of filtration.

5) Appurtenances

- The agitation of sand grains during washing of filters is carried out either by compressed air or by water jet. When air is to be used, an air compressor of required capacity should be installed.

- The dirty water after washing of filter is collected in wash-water troughs or gutters, which are placed above sand bed level.

Advantages

1. Turbidity water may be treated.

2. Land required is less compared to slow sand filter.

3. Operation is continuous.

Disadvantages

1. Requires skilled personnel for operation and maintenance.

2. Less effective in bacteria removal.

3. Operational troubles.

2) PRESSURE TYPE RAPID SAND FILTERS

A pressure filter is a small rapid gravity sand filter placed in a closed cylindrical vessel & through which the water to be treated is passed under pressure which is higher than atmospheric pressure. So it should be noticed that the filters located in air tight vessels. The raw water is generally pumped into the vessel by means of a pump. The pressure normally developed may 25 to 65   meter     head of water.

The pressure filter can have rate of filtration 2 to 5 times higher than a rapid sand filter, lower rate of filtration may be used for domestic, industrial supplies &the high rate is used for swimming pool  recirculating  supplies.  Their rate of filtration normally ranges from 6000 to 15000 liters per hour per sq.m of filter area

Considering the water quality this is not efficient than the rapid sand filter, in removing bacteria& turbidities. The quality of filter water is very poor considering for consumption of public supply.

The pressure filters are classified as;

- Vertical pressure filters.

- Horizontal pressure filters.

Advantages

1. It is compact and automatic operation.

2. These are ideal for small estates and small water works.

3. These filters require small area for installation.

4. Small number of fittings are required in these filters.

5. Filtered water comes out under pressure no further pumping is required.

Disadvantages

1. Due to heavy cost on treatment, they cannot be used for treatment large quantity of water.

2. Proper quality control and inspection is not possible because of closed tank.

3. The efficiency of removal of bacteria and turbidity is poor.

4. Change of filter media, gravel and repair of drainage system is difficult.

DISINFECTION OF WATER

The process of killing pathogenic bacteria (disease producing bacteria) is known as disinfection or sterilization of water. The chemical or substances which are used for killing the bacteria are .known as disinfectants. After filtration water goes to the disinfection plant before public supply.

Necessity;

- In the water treatment processes like sedimentation, coagulation, etc., all the bacteria from the water cannot be removed.

- The filtration of water removes the suspended impurities & removes a larger percentage of bacteria but still some of the bacteria remain there in the filtered water.

- Moreover there is even chance of getting the water contaminated during its flow through the water distribution system especially in case of intermittent supply, where the pipes remain empty for a considerable period.

Therefore, in order to ensure safety to such water it is disinfected or sterilized.

Different methods are adopted for disinfection by using some chemicals. But  it  has been found that the chlorine is the most useful & the best chemical for disinfection.

Requirements of good disinfectants:

- They should destroy all the harmful pathogens & make it safe for use.

- They should not take more time i n killing bacteria.

- They should act rapidly.

- They should be economical & easily available.

- They should not make the water poisonous.

- The concentration should be determined by simply & quickly.

- They should be easy to handle, easy to store &easy to carry.

- Their lasting period in the disinfection chamber should be fairly good, so that they do not allow any further grown of bacteria.

- The disinfectant does should be such that, it may leave some residual concentration for protection against contamination in the water.

- It should not change the subsequent in water.

- They must be capable of being dissolved in water.

The factors affecting the efficiency of disinfection;

- Type, condition & concentration of organisms to be destroyed.

- Type & concentration of disinfectant.

- Contact time & concentration of disinfectants in water.

- Chemical & physical characteristics of water to be trated.

- Temperature of water.

Methods of disinfection

Following are some of the methods of disinfection

- Boiling of water

- Ultraviolet rays

- Chlorination

- Potassium permanganate

- Ozone

- Excess lime treatment

Out of the above, treatment with chlorine is the most popular & economically effective.

CHLORINATION

Chlorination is the addition of chlorine to kill the bacteria. Chlorination is very widely adopted in all developing countries for treatment of water for public supply. Chlorine is available in gas, liquid or solid form (bleaching powder).

Advantages of chlorine

1. Chlorine is the most powerful for destroying the bacteria absolutely.

2. The process of application is easy

3.  It can be stored easily.

4. Chlorine is manufactured easily by electrolytes of common salts (NaCI).

5. Dosage can be controlled precisely.

6. Dose not form harmful constituents on reaction with organics of inorganics in water.

7. It leaves required residue in water to neutralize recontamination later.

8. It can be easily detected by simple orthotolidine test.

9. The optimum does can be easily found out by break point chlorination

Precautions in using chlorine

Chlorine gas or liquid is highly corrosive & lethal to Inhale. Hence it is to be stored carefully in sealed container at a distance.

If the water contains phenol compounds, there is a reaction with chlorine can result in cancer causing substances.

Application of chlorine

This may be applied by the following methods;

1. Dry chlorine gas; It may not be drawn from the liquid chlorine; it is applied directly to the water supply through Submerged diffusers. This method is not satisfactory because it results corrosion in pipes.

2. Chlorine solution; The chlorine solution formed by mixing chlorine gas with small quantity of water is applied to the water supply through a discharge line. This method is very commonly used.

3. Powder form;     In the form of hypochlorite or bleaching powder. Bleaching powder is commonly used for smaller water –plants.

Chlorine is mixed with water with a device which is known as chlorinator. These are various types of chlorinators available. A simple type is described below:

It consists of a glass ja containing distilled water in which the chlorine is fed from the chlorine cylinder. An agitator is provided inside the jar as shown in fig, 6.21. It agitates the distilled water to form a uniform chlorine solution. This solution is fed to the water in the main pipe through injector pipe. To control the dose of chlorine a regulator is provided in the injector pipe. One regulator is provided in chlorine inlet pipe & the other is provided in distilled water pipe to regulate the flow of chlorine & distilled water respectively.

Break point chlorination

This term gives an idea of the extent of chlorine added to water. It represents a dose of chlorination beyond which any further addition of chlorine will appear as free residual chlorine.

Advantages of Break point chlorination;

- It destroys all the disease bacteria

- It removes completely taste and odours from the water.

- It completely oxidizes impurities of water e.g. ammonia, etc.

- It helps in removing colour or water which is due to organic matters.

- It also prevents weeds growth in water.