Velocity Ratio , Slip and creep  of Belt :

Velocity Ratio  of  Belt :  It can be defined as the speed of the driven pulley to the speed of the driving pulley.

Let  d1= Dia. Of the driving pulley.

d2= Dia. Of the driven pulley

N1=Speed of  the  driving pulley in rpm.

N2=Speed of  the  driven pulley in rpm.

Length of the belt passes over the driving pulley in one minute = π d1 N1

Similarly , the length of the belt passes over the driven pulley in one minute= π d2 N2

Since the length of the belt passes over the driving pulley in one minute  is equal to the length of the belt passes over the driven pulley in one minute, therefore

π d1 N1 = π d2 N2

Velocity ratio =    N2N1 =  d1d2

If the thickness of the belt (t) is considered , then the velocity ratio is given by

Velocity ratio, =   N2N1 =  d1+td2+t

Velocity Ratio of compound Belt Drive :

Compound Belt Drive

Let  d1, d2, d3  and  d4 = Diameters of pulleys 1 ,2,3 and 4, respectively.

N1 , N2, N3 and  N4 = Speed of the pulleys 1 ,2,3 and 4, respectively .

We know that , velocity ratios Velocity ratio =    N2N1 =  d1d2 and    N4N3 =  d3d4

N2N1 X N4N3 =  d1d2  X  d3d4

N4N1= d1d2 x d3d4                  ( N2 =N3 as they are connected to the same shaft)

Or    Speed of last  driven pulley Speed of first driver pulley = Product of dia.of driver pulleysProduct of dia.of driven pulley

Slip of the Belt : In the  belt drive , it is assumed that a firm grip is existing between the belt and pulley , but sometimes , this frictional grip becomes insufficient. This may cause some forward motion of the driver pulley without carrying the belt with it or forward motion of the belt without carrying the driven pulley with it.This phenomenon is known as slip of the Belt and it is generally expressed in percentage.

The effect of slip is to decrease the velocity ratio of the system. Therefore , the belt drive should not be used where a definite velocity ratio is very important.

Let , S1% =  %  of slip between the driving pulley and the belt.

S2% =  %  of slip between the driven  pulley and the belt.

Velocity ratio    N2N1 =  d1d2 (1- S100 ) ---  Where  S= S1+ S2 = total percentage of slip.

When the thickness of the belt (t ) is considered , then

Velocity ratio, =   N2N1 =  d1+td2+t (1- S100 )

Creep of the Belt : When the belt moves from the slack side to the tight side to transmit the power, a certain portion of the belt extends and it again contracts when the belt moves from the tight side to the slack side. This is because , the belt material is elastic , it elongates more on tight side than on slack side, as the tension in the tight side is more than the tension in the slack side. Due to these changes of length , there is a relative motion between the belt and pulley surfaces. This relative motion is known as creep of belt.

The total effect of creep is to reduce the speed of the driving pulley or driven pulley .

Let, σ1 and σ2 = Stresses in the belt on the tight side and slack side respectively.

E= Young’s modulus for  the belt material

Then , by considering creep , the velocity ratio is given by,

N2N1 =  d1d2 x E+√σ2E+√σ1

Length of the Belt  (Open Belt Drive) : An open belt drive is shown in fig , in which both the pulleys rotate in the same direction.

Length of the Belt  (Cross Belt Drive) :

Ratio of Driving Tensions :

Ratio of Driving Tensions

Let us consider a driven pulley rotating in the clockwise direction in clockwise direction as shown in fig.

Let T1 = Tension in the tight side

T2 = Tension in the slack side

θ= Angle of contact of the belt over the pulley in radians

μ= Coefficient of friction between the belt and the pulley.

Then ,    T1T2 = eµӨ

Or     2.3 log ( T1 T2) = μθ

For V- Belt and Rope drives , the ratio of driving tensions is given by ,

loge( T1 T2) = μθsinβ= μθ Cosec β

where  2β= Angle of the groove.

Centrifugal Tension : When the belt runs continuously over the pulleys , some centrifugal force is developed due to its own weight , whose effect is to increase the tension on both

tight and slack sides.This tension caused by the centrifugal force known as centrifugal tension.

Let ,      m= mass per unit length of the belt in kg.

TC= centrifugal tension on tight and slack sides of the belt element in Newton (N)

v= velocity of the belt in mtr/sec.

r=  radius of the pulley in mtr.

Then , Centrifugal tension  TC= mv2

When the centrifugal tension is taken in to account , then total tension in the tight side is

given by,               Tt1 = T1 + Tc

and                        total tension in slack , Tt2 = T2+ Tc

Power transmitted  P=  (T1 - T2)V  ------ Kw

Initial Tension in the Belt :

When the belt is fitted over the pair of pulleys , the belt ends are joined firmly together, so that it has to move continuously over the pulleys , since the motion of the belt from the driving pulley and driven pulley is governed by the firm grip due to the friction between the belt and pulleys. Inorder to increase this grip , the belt is tightened up. At this stage , even though the pulleys stationary , the belt is subjected to some tension , this tension is known as initial tension. Let   T0= Initial tension in the belt

T1= Tension in the tight side of the belt

T2= Tension in the slack side of the bel ,

α = Coefficient of increase in length of the belt per unit force.

Therefore , T0 = T1+T22

Initial tension with consideration of centrifugal tension is given by,

T0 = T1+T2+2Tc2

Where Tc = centrifugal tension.

Maximum Tension in the Belt :  The maximum tension in the belt ,

T=  Maximum stress  x cross sectional area of the belt.

T=      σ.  b.  t

When the centrifugal tension is neglected , then maximum tension in the belt ,

T= T1

When the centrifugal tension is considered ,then

T= T1 + TC

------------------------------------------------

Power  transmitted  by the  Belt ( Flat and  V belt) and Rope :

1.The power transmitted by the flat belt is given by :

P=(T1 –T2) V             in   N-mtr/sec  or watts.

Where ,   T1= Tension on tight side in Newton.

T2= Tension on slack side in Newton.

V =  Velocity of the belt in  mtr/sec

Maximum Power Transmitted by Belt :

For maximum power transmission , centrifugal tension in the belt  must be equal to one third of the maximum allowable belt tension ,

Note : For maximum power ,   T1=  T - Tc  = T- T3  = 23 T

and velocity of the belt : Vmax = √T3m

Problems on Belt Drive :

- An  engine , running at 150 rpm , drives a  line  shaft  by means of a belt.The  engine pulley is 750 mm diameter and the pulley on the line shaft being 450 mm. A 900 mm diameter pulley on the line shaft drives a 150 mm diameter pulley keyed to a dynamo shaft. Calculate the speed of the dynamo shaft , when

- There is no slip

- There is a slip of 2% at each drive.

Answer : Data given,   N1= 150 rpm ;   d1= 750 mm,  d2=450 mm,   d3= 900mm ,  d4=150mm,

1.when there is no slip :        N4N1= d1d2 x d3d4

N4150= 750450 x 900150   =10

N4= 150 X10 = 1500 rpm.

2.When there is a slip of 2% at each shaft :            N4N1  =  d1d2  X  d3d4 ( 1 - S1100 ) (1 - S2100 )

N4N1  =  d1d2  X  d3d4 ( 1 - S1100 ) (1 - S2100 )

N4150  =  750450  X  900150 ( 1 - 2100 ) (1 - 2100 )

N4    150  = 9.6,   N4 = 9.6 x150 = 1440 rpm .

Problem 2. Calculate the power transmitted by a belt running over a pulley of 600 mm diameter at 200 r.p.m. The coefficient of friction between the belt and the pulley is 0.25, angle of lap is 1600  and maximum tension in the belt is 2500 N.

Answer : Data: d=600mm  = 0.6 m ,  N = 200 rpm , μ = 0.25,

θ= 1600  = 160 * π180 = 2.793 radians , T1 =  2500 N

V = π*d*N60 = π*0.6*20060 = 6.284 m/sec.

loge (T1T2) = μθ

= 0.25 * 2.793    = 0.6982.

loge (T1T2) = 0.6982

(T1T2) = e(0.6982)  = 2.01

T2 = T12.01 = 25002.01 = 1244 N.

Power transmitted by the belt = P = (T1 – T2) V

= (2500 - 1244) * 6.284

= 7890 W or 7.89 kW.

Note:  For open belt drive ,  Sinα = r2-r1x ,   and   Angle of contact, θ=( 1800 - 2α )

For crossed  belt drive ,  Sinα = r1+r2x ,  and  Angle of contact, θ=( 1800 + 2α )

Problem .3. Two pulleys , one 450 mm diameter and the other 200 mm diameter are on parallel shafts 1.95 apart and  are connected by a crossed belt. Calculate the length of the belt required and the angle of contact  between the belt and each pulley. What power can be transmitted by the belt when larger pulley rotates at 200 rpm , if the maximum permissible tension in the belt is 1 KN , and the coefficient of friction between the belt and pulley is 0.25.

Answer :   Data,        d1=450 mm=0.45 mtr,             r1=0.225 mtr

d2=200 mm=0.2 mtr,                r2=0.1 mtr ,      x= 1.95 mtr,  N1= 200 rpm

T1= 1 KN  = 1000 N ,              μ= 0.25

Velocity of the belt ,  V =  π*d1*N160      mtr/sec

V =  π*0.45*20060 = 4.714 mtr/sec.

Length of cross the belt :       L =  πr1+r2+2x+(r1+r2)2x

=  π0.225+0.1+2*1.95+(0.225+0.1)21.95

= 4.975 mtr.

Angle of contact :               Sinα = r1+r2x    (For crossed  belt drive)

Sinα = 0.225+0.11.95    = 0.1667

α = sin-1(0.1667) =9.60

Therefore ,                                        θ=( 1800 + 2α )  (For crossed  belt drive)

θ=( 1800 + 2  *9.6 )  =199.20 = 199.2* π180  = 3.477 radians.

Power transmitted :                             loge (T1T2)  =  μθ

= 0.25 * 3.477 =0.8692

Therefore ,                  (T1T2) = e(0.8692)  = 2.385

T2 = T12.385 = 10002.385 = 419.28 N.

Power ,  P = (T1 – T2 ) V    ---- Watts

P = (1000 – 419.28 ) *4.714

= 2737.5 Watts . =2.737 kw.

------------------------------------------------------

Problem 4.  A shaft rotating at 200 rpm. Drives another shaft at 300 rpm and transmits  6 Kw  through a belt. The  belt is 100 mm wide and 10 mm thick. The distance between the shafts is 4 mtr .  The smaller  pulley is  0.5 mtr in diameter. Calculate the stress in the belt , if it is  open belt drive . Take   μ= 0.3.

Answer : N1= 200 rpm ; N2 = 300 rpm ; P=6 KW =6x1000 = 6000 Watts.   b=100 mm;  t=10 mm;  x = 4 mtr ;

d2=0.5 mtr ;     μ= 0.3

Velocity of the belt ,    V =  π*d2*N260      mtr/sec  =  π*0.5*30060      = 7.855 mtr/sec

But,	  N2N1 =  d1d2                d1=   N2N1 * d2 =   300200 * 0.5 =0.75 mtr.              r1= 0.375 mtr

For open belt drive ,  Sinα = r2-r1x ,   and   Angle of contact, θ=( 1800 - 2α )

but         Sinα = r1-r2x =   0.375-0.254 =  0.03125              α = sin-1(0.03125) =1.80

Angle of contact, θ=( 1800 - 2α ) =180 – 2 * 1.8 = 176.40 = = 176.4 * π180  = 3.08 radians.

loge (T1T2)  =  μθ

loge (T1T2)  =  μθ = 0.3 *3.08 = 0.924

(T1T2) = e(0.924)  = 2.52

T1= 2.52 * T2 ----------------------------------(1)

Power transmitted  ,  P = (T1 – T2 ) V    ---- Watts

6000 = (2.52 * T2 - T2) * 7.855

= T2(2.52 – 1 ) * 7.855 = 11.9396 T2

T2 = 600011.9396 =502.5 N

But ,          T1= 2.52 * 502.5 = 1266.3 N

Maximum stress in the belt  , T1=      σ.  b.  t

1266.3=  σ*  100 *  10

σ = 1266.3100*10 = 1.266  N/mm2

------------------------------

Problem . 5. A leather belt is required to transmit 7.5 kw from a pulley 1.2 mtr in diameter, running at 250 rpm. The angle embraced is 1650 and the coffficient of friction between the belt and the pulley is 0.3. If the safe working stress for the leather belt is 1.5 MPa , density of leather 1 Mg/m3 and thickness of the belt 10 mm, determine the width of the belt taking centrifugal tension in to account.

Answer : Power  P= 7.5 kw = 7500 watts ,             d= 1.2 mtr ,                 N= 250 rpm,

θ= 1650 = 165* π180 = 2.88 radians ,    μ= 0.3,     σ = 1.5 MPa = 1.5 x 106 N/mm2

ρ = 1 Mg/m3 = 1 x 106 g/m3= 1000  kg/m3 ,

V= μ*d*N60 = μ*1.2*25060 = 15.71 m/sec

Power transmitted ,                            P = (T1 – T2 ) V

7500 = (T1 – T2 ) 15.71

T1 – T2 = 750015.71 = 477.4 N -----------------------(1)

logeT1T2 = μ θ

logeT1T2 = 0.3 X 2.88 = 0.864

Therefore,  T1T2 =e(0.864) = 2. 373

T1 = 2.373 x T2

Substitute the value of T1 in equation (1)

2.373 T2 - T2 = 477.4

T2( 2.373  - 1 ) = 477.4

T2 = 477.41.373 = 347.7 N

And , T1 = 2.373 x 347.7 = 825 N

Mass of the belt / metre length  = Area x Length x Density

= b x t x l x ρ

= b x 0.01 x 1 x 1000 = 10 b kg.

Centrifugal tension , Tc = m v2 = 10 x b b(15.71)2

= 2468 b  newton

Maximum tension in the belt ,  T= σ x b x t  = 1.5 x 106 x b x 0.01 = 15000 b   Newton

But , T = T1 + Tc

15000 b = 825 + 2468 b

( 15000 – 2468 ) b = 825

b = 82512532 = 0.0658 mtr -----------------------------------------------------

Problem .6.  A rope drive transmits 600 kw from a pulley of effective diameter 4 mtr which runs at a speed of 90 rpm . The angle of lap is 1600  the angle of  groove 450 , the coefficient of friction 0.28 , the mass of the rope is 1.5 kg/mtr and the allowable tension in each rope 2400 N. Calculate the number of ropes required.

Answer :         Given ,     P = 600 KW ,        d = 4 mtr ,    N = 90 rpm. ,    θ= 1600 = 160* π180 =  2.8 rad

2β = 450  ,     β = 22.50  ,    μ= 0.28,      m =1.5 kg/mtr ,   T=2400 N

Velocity of rope ,                V= π*d*N60 = π*4*9060 = 18.85 m/sec

Centrifugal tension , Tc = m v2 = 1.5 x (18.85)2 =533N

When the centrifugal tension is considered ,then

T= T1 + TC  or  T1 = T - TC = 2400-533 =1867 N

But ,       logeT1T2 = μθsinβ = 0.28* 2.8sin⁡(22.5)  =2.05

Therefore,               T1T2 = e(2.05) = 7.768

T2 = 18677.768 = 240.34 N

Power transmitted per rope ,              P = (T1 – T2 ) V  = (1867 -240.34) *18.85 =30662 watts or 30.662 Kw.

Number of ropes =   Total power transmittedPower transmitted per rope   = 60030.662 = 19.5 or  say 20 ropes

Problem 7. : Determine the width of a 9.75 mm thick leather belt required to transmit 15 kW from a motor running at 900 rpm. The diameter of the driving pulley of the motor  is 300 mm. The driven pulley runs at 300 rpm and the  distance between the centers of two pulleys is 3 meters. The density of the leather is 1000 kg/m3. The maximum allowable stress in the leather is 2.5 MPa . The coefficient of friction between the leather and pulley is 0.3. Assume open belt drive and neglect the sag and slip of the belt.

Answer :    Given :  t=9.75 mm =9.75 x10-3 mtr ,    P = 15kW= 15 x103  watts ,     N1 = 900 rpm ,

d1 = 300 mm = 0.3 mtr ,  N2 = 300 rpm ,   x = 3 mtr ,     ρ = 1000 kg/m3 ,

σ  = 2.5 MPa = 2.5 x106  N/m2 ,         μ= 0.3 ,

But,	  N2N1 =  d1d2

d2 =   N1N2*d1  =    900300*0.3= 0.9 mtr

and ,                           V= π*d*N60 = π*0.3*90060 = 14.14 m/sec

Note:  For open belt drive ,  Sinα = r2-r1x ,   and   Angle of contact, θ=( 1800 - 2α )

Sin α  = r2-r1x  = d2-d12x = 0.9-0.32*3  =0.1

α = sin-1(0.1) =5.740

Angle of contact, θ=( 1800 - 2α ) = 1800 - 2 * 5.740  = 168.520

= θ= 168.520 = 168.52* π180 = 2.94 radians ,

logeT1T2 = μ θ= 0.3 x 2.94 = 0.882

Therefore,               T1T2 = e(0.882) = 2.42

T1 = 2.42 x T2 --------------(1)

Power  transmitted  ,              P = (T1 – T2 ) V  = 15 x103  =(2.42 T2 - T2 ) x 14.14

= 15 x103 = T2  (2.42 - 1 ) x 14.14

=  20.07 T2

T2 =  15 x103  / 20.07 = 747.4 N

And                                          T1 = 2.42 x T2

T1 = 2.42 x 747.4 =1808.7 N

Mass of belt per mtr length , m = b  x t  x l  x ρ

= b x 9.75 x10-3 x 1 x 1000

= 9.75 b kg

And  centrifugal tension , Tc = m v2 = 9.75 x b x (14.14)2

= 1950 b   N

Maximum tension in belt ,   T = σ x b x t

=2.5 x 106 x b x 9.75 x10-3

= 24400 b  N

But ,     T= T1 + TC

24400 b = 1808.7 + 1950 b

b ( 24400 – 1950 ) = 1808.7

b=  1808.7 22450 = 0.080 mtr = 80 mm

Problem . 8. A pulley is driven by a flat belt , the angle of lap being 1200. The belt is  100 mm wide by 6 mm thick and density 1000 kg/m3. If the coefficient of friction is 0.3 and the maximum stress in the belt is  not to exceed 2 MPa .  Calculate the greatest power which the belt can transmit and the corresponding speed of the belt.

Answer :                          θ= 1200 = 120* π180 = 2.1 radians ,  b = 100 mm  = 0.1 m  ,  t = 6 mm = 0.006 m  ,

ρ = 1000 kg/m3  ,     μ= 0.3,      σ = 2 MPa = 2 x 106 N/ m2

Maximum tension in the belt , ,  T= σ x b x t  = 2 x 106 x 0.1 x 0.006 = 1200   Newton

Mass of the belt per meter length  , m = b x t x l x ρ =0.1 x 0.006 x 1 x 1000 = 0.6 kg/min

Speed of the belt for maximum power ,  V= √T3 m =  √12003 *0.6 = 25.82m/sec

For maximum power to be transmitted ,  Tc = T3 = 12003 = 400 N

Tension on  tight side of the belt ,  T1= T - Tc

= 1200 – 400 = 800 N

Now , 		         logeT1T2 = μ θ

=0.3 x 2.1 = 0.63

T1T2 =e(0.63) =  1.88

T2= T11.88 =  8001.88 = 425.5 N

Greatest power which the belt can transmit , P = (T1 – T2 ) V  = (800 – 425.5 ) x 25.82 =9670 watts = 9.67 kW

Introduction to Gear Drive : The gears are toothed wheels used for the transmission of motion from one shaft to another or between a shaft and a slide. This is accomplished by a successively enganging the tooth. Gears use no intermediate link or connector and transmit the motion by direct contact.

While transmitting the motion or power between the two shafts, the effect of slipping is to reduce the velocity ratio of the system. In precision machines , where a definite velocity ratio is very essential , in such case the transmission of motion or power is carried out by means of gears. Gear drives are generally used where the centre between the driver and the follower is very small and requires positive drive.  A simple gear drive is shown in fig.

Advantages and Disadvantages of Gear Drives :

Advantages ;

- Positive velocity ratio.

- Used for transmitting large power.

- High efficiency.

- Reliable service.

- Compact in layout.

- Transmission of power between shafts of small centre distance.

Disadvantages :

- Gear manufacturing requires special tools and requirements.

- Error in tooth cutting create vibrations and noise during operation.

- High production cost.

Classification of Gears :

According to the position of the shaft axes, there are 3 classification

- Parallel

- Intersection

- Non-parallel & Non-intersecting

Based on the engagement of the gear, there 3 types of gears. they are

- External Gearing

- Internal Gearing

- Rack and pinion

The classification of Gears according to the velocity of gears are 3 types

- Low velocity

- Medium velocity

- High velocity

Based on the tooth profile on the gear surface , there 3 types of gears. they are

- Gears with straight teeth : Spur gears

- Gears with curved teeth : Helical gears

- Gears with inclined teeth : Spiral gears

Spur Gear Terminology:

Pitch circle: It is an imaginary circle which by  pure  rolling action , would give the same motion as the actual gear.

Addendum:  It is a radial distance of a tooth from the pitch circle to the top of the tooth.

Dedendum: It is a radial distance of a tooth from the pitch circle to the bottom  of the tooth.

Spur gear terminology.

Circular pitch : It is the distance measured on the circumference of the pitch circle from the point of one teeth to the  corresponding point on the next tooth . It is denoted by ‘Pc’.

Pitch circle , Pc = π dT

Where ,  d = Dia . of pitch circle.

T = No. of teeth on the gear.

If two gears are meshing correctly and their diameters are  d1 and  d2 and their teeths are T1  and  T2

Then , Pc = π d1T1  = π d2T2 or  d1     d2  =   T1     T2

Diametral pitch : It is the ratio of number of teeth to the pitch circle diameter in mm. It is denoted by ‘Pd’

Diametral pitch , Pd =  T    d

=  π Pc                 	Pc=π dT

Where  ,   T = No. of teeth on the gear.

d =  Dia . of pitch circle.

Module : It is the ratio of pitch circle diameter in mm to the number of teeth . It is denoted by ‘m’

Module , m =  dT

Gear Trains :  When the two or more gears are made to mesh with each other to transmit power  from one shaft to another , then such combination is known as gear train. The type of gear trains used depends upon the velocity ratio required and the relative position of the axis of the shaft.

Types of Gear Trains : Following are the types of gear trains :

  - Simple gear train.

  - Compound gear train.

  - Reverted gear train.

  - Epicyclic gear train.

Simple gear train:

Simple gear train :	 When there is only one gear on each shaft, as shown in Fig. , it is known as simple gear train. The gears are represented by their pitch circles. When the distance between the two shafts is small, the two gears 1 and 2 are made to mesh with each other to  transmit motion from one shaft to the other, as shown in Fig.  Since the gear 1 drives the gear 2, therefore gear 1 is called the driver and the gear 2 is called the driven or follower. It may be noted that the motion of the driven gear is opposite to the motion of driving gear.

Let N1 = Speed of gear 1(or driver) in r.p.m.,

N2 = Speed of gear 2 (or driven or follower) in r.p.m.,

T1 = Number of teeth on gear 1,

and        T2 = Number of teeth on gear 2.

Since the speed ratio (or velocity ratio) of gear train is the ratio of the speed of the driver to the speed of the driven or follower and ratio of speeds of any pair of gears in mesh is the inverse of their number of teeth,

Therefore,                                                       Speed ratio  = N1N2 = T1T2

It may be noted that ratio of the speed of the driven or follower to the speed of the driver is known as train value of the gear train.

Mathematically,            Train value  = N2N1 = T1T2

Compound Gear Train :   When there are more than one gear on a shaft, as shown in Fig. 13.2, it is called a compound train of gear. We have seen  that the idle gears, in a simple train of gears do not effect the speed ratio of the system. But these gears are useful in bridging over the space between the driver and the driven.

But whenever the distance between the driver and the driven or follower has to be bridged over by intermediate gears and at the same time a great speed ratio is required, then the advantage of intermediate gears is intensified by providing compound gears on intermediate shafts. In this case, each intermediate shaft has two gears rigidly fixed to it so that they may have the same speed. One of these two gears meshes with the driver and the other with the driven or follower attached to the next shaft as shown in Fig.

In a compound train of gears, as shown in Fig. the gear 1 is the driving gear mounted on shaft A, gears 2 and 3 are compound gears which are mounted on shaft B. The gears 4 and 5 are also compound gears which are mounted on shaft C and the gear 6 is the driven gear mounted on shaft D.

Let N1 = Speed of driving gear 1,

T1 = Number of teeth on driving gear 1,

N2 ,N3 ..., N6 = Speed of respective gears in r.p.m.,

and T2 ,T3 ..., T6 = Number of teeth on respective gears.

Since gear 1 is in mesh with gear 2,  therefore its  speed ratio  is

Speed ratio  = N1N2 = T2T1   -------------------------(i)

Similarly, for gears 3 and 4,   speed ratio is

Speed ratio  = N3N4 = T4T3  -------------------------(ii)

and  , for gears 5 and 6, speed ratio is

Speed ratio  = N5N6 = T6T5  -------------------------(iii)

The speed ratio of the compound gear train is given by multiplying the equations (i), (ii) and (iii),

N1N2 x   N3N4 x  N5N6 = T2T1  x  T4T3  x  T6T5  or   N1N6 =  T2T1 x T4T3  x  T6T5

Or                   Speed ratio = Speed of the first driverSpeed of the last driven

Train value = Speed of the last  drivenSpeed of the first  driver

The main advantage of compound train over a simple gear train is that a much larger speed reduction from the first shaft to the last shaft can be obtained with small gears.

Reverted  Gear Train : When the axes of the first gear (i.e. first driver) and the last gear (i.e. last driven or follower) are co-axial, then the gear train is known as reverted gear train as shown in Fig. We see that gear 1 (i.e. first driver) drives the gear 2 (i.e. first driven or follower) in the opposite direction. Since the gears 2 and 3 are mounted on the same shaft, therefore they form a compound gear and the gear 3 will rotate in the same direction as that of gear 2. The gear 3 (which is now the second driver) drives the gear 4 (i.e. the last driven or follower) in the same direction as that of gear 1. Thus we see that in a reverted gear train, the motion of the first gear and the last gear is same.

Then , Speed ratio = N1N4 = =  T2T1 x T4T3

Speed ratio = Product of number of teeth on drivensProduct of number of teeth on drivers

Epicyclic Gear Train : In an epicyclic gear train, the axes of the shafts, over which the gears are mounted, may move relative to a fixed axis. A simple epicyclic gear train is shown in Fig.  where a gear A and the arm C have a common axis at O1 about which they can rotate. The gear B meshes with gear A and has its axis on the arm at O2 , about which the gear B can rotate. If the arm is fixed, the gear train is simple and gear A can drive gear B or vice- versa, but if gear A is fixed and the arm is rotated about the axis of gear A (i.e. O1), then the gear B is forced to rotate upon and around gear A. Such type of  motion is called epicyclic .

The epicyclic gear trains are useful for transmitting high velocity ratios with gears of moderate size in a comparatively lesser space. The epicyclic gear trains are used in the back gear of lathe, differential gears of the automobiles, hoists, pulley blocks, wrist watches etc.

Problems :

1.Two parallel shafts , about 600 mm apart are to be connected by spur gears. One shaft is to run at 360 r.p.m. and the other at 120 r.p.m. Develop the gears , if the circular pitch is to be 25 mm.

Answer :                  Given :   x= 600 mm ,     N1 = 360 r.p.m.    N2 = 120 r.p.m.    Pc = 25 mm

We know that ,                     V.R. =	  N2N1 =  d1d2   ,   d1d2 = 120360 = 13  ,    d2 = 3 d1

But ,       x = d1+d22  ,       d1 + d2 = 2 x           =  2*600=1200 mm  ,

Substitute the value of ,’ d2’  then    d1 + 3d1 =1200    ,

4d1 = 1200 ,

d1=  12004 =300 mm

and  ,            d2 = 3 x 300 = 90

∴         Pc = π d1T1

∴        T1=	π d1Pc

T1=	π*30025 =37.7 ≅ 38

And , ∴        T2= π d2Pc  = π*90025  =113.1 ≅ 114

Since the values T1 and T2 are rounded off to the nearest values , therefore gear diameters have to be recalculated,

∴         Pc = π d1T1    ,    d1= Pc x T1 π

d1 =   Pc x T1 π       =        25 x 38 π      =302.4 mm

d2 =   Pc x T2 π       =        25 x 114 π     = 907.18 mm

The centre distance , 	,                           x =  d1+d22

x =  302.4+907.182 =604.79 mm

Problem . 2  Two parallel shafts are to be connected by spur gearing . The approximate distance between the shafts is 600 mm . If  one shaft runs at 120 r.p.m and the other at 360 r.p.m.  Calculate the number of teeth on each wheel , if the module is 8 mm. Also determine the exact distance apart of the shafts.

Solution : Given :  x = 600 mm , N1 = 120 r.p.m.    N2 = 360 r.p.m.    m = 8 mm

V.R. =	  N2N1 =  d1d2   ,

d1d2 = 360120 =3

d1 = 3 d2

But ,       x = d1+d22  ,       d1 + d2 = 2 x

d1 + d2 = 2 x     ,   = 2 x 600 = 1200

3d2 + d2 = 1200

4d2 = 1200

d2 = 12004  = 300 mm

and  					d1 = 3 d2

d1 = 3 x 300 = 900 mm

module , m =  d1 T1    ,  T1= d1m

T1= 9008  =112.5 say 114

And 			               m =  d2 T2    ,    T2= d2m

T2= 3008  = 37.5 say 38

Since the values T1 and T2 are rounded off to the nearest values , therefore gear diameters have to be recalculated,

∴   m =  d1 T1

d1 = m T1

d1 = 8 x 114 = 912 mm

∴   m =  d2 T2

d2 = m T2

d2 = 8 x 38 = 304 mm

The centre distance , 	,                           x =  d1+d22

x =  912+3042 =608 mm

Problem .3.  The number of teeth of a spur gear is 40 and it rotates at 220 rpm . What will be the circular pitch line velocity of it has module of 2 mm ?

Solution  :   Given:   T = 40  ,  N = 220 R.P.M. , m = 2 mm

∴         Pc = π dT

=  π m

=  π x 2 = 6.283 mm

We know that ,                                    m =  d T

d = m x T

d = 2 x 40 = 80 mm = 0.08 mtr

Pitch line velocity ,                        V= π*d*N60 = π*0.08*22060 = 0.921  m/sec